|  | UNIVERSIDADE FEDERAL DO PARÁ INSTITUTO DE TECNOLOGIA PROGRAMA DE PÓS-GRADUAÇÃO EM ENGENHARIA ELÉTRICA DISCIPLINA: TRANSMISSÃO DIGITAL – PROF. ALDEBARO KLAUTAU |
| --- | --- |

## Laboratory:
AM broadcast transmission with USRP

Goal: Familiarize the reader with a signal generator, spectrum analyzer and concepts of AM transmission. Provide the first contact with a USRP.

In summary, the task consists in generating in Matlab two WAV (Microsoft RIFF, linear PCM, 16 bits per sample) files, then multiplexing them into an FDM with AM signals, store in a complex binary file, read this file using the provided GNU Radio Companion .grc file, which will then use a USRP to transmit it. The signals must be captured by an off-the-shelf broadcast AM radio, which will be also provided. Besides, the signal must be captured by a spectral analyzer, and the student must identify aspects such as power and bandwidth. This last part will be conducted at LASSE / UFPA with help from the tutor.

Preparation:

- Read the manuals of the spectrum analyzer and signal generator available at LASSE. It is also suggested the reading of associated application notes (e.g., about spectrum analysis by Agilent, cited in the textbook).

- After reading, participate of a hands-on training practice at LASSE conducted by the tutor.

- Learn how to generate an AM signal with the signal generator

- Learn how to identify the strongest AM stations at LASSE

Task 1: Identify the strongest AM stations at LASSE. Based on this data, choose two RF frequencies to locate your own “AM stations”. One restriction is that they must be separated by a minimum of 30 kHz, Talk to your colleagues: another restriction is that nobody can have the same pair of RF frequencies. Knowing how to work in group is always important.

Task 2: Create WAV files with two signals. Then organize these signals using an FDM scheme and save them into one binary file that supports complex signals. An example of script to save a file in the required format is listed in the appendix. Use Matlab and a sampling frequency of Fs=250 kHz. Assume the USRP will use a carrier frequency of 1.5 MHz, which will correspond to a shift of your signal from [-Fs/2, Fs/2[ to 1500000+[-Fs/2, Fs/2[. Suggested steps:

- Create two signals, s1 and s2, of 5 kHz bandwidth and 20 seconds of duration, each. Use one song and one recording of your own voice, as indicated in the previous Laboratory. Store them as WAV files.

- Use frequency upconversion and locate them within the bandwidth you have available. Separate them by at least 30 kHz, but pay attention that at RF they will be located where you want to.

- Save the FDM signal in the file format used by GNU Radio, such that it can be read by the script read_complex_binary.m in the appendix. In summary, use a headerless, little-endian, with two “floats” (4 bytes each) per complex sample, one for the real and the other for the imaginary part. There is a script in the appendix to write such file. Use an extension .bin or .raw.

- Report your design: modulation index, bandwidth and any other value you consider important.

- As a sanity check, plot the PSD of your FDM signal and include in the report. Properly label the graph.

- Demodulate and listen to it. Correct eventual mistakes and have everything set to be tested at the lab.

Task 3: Understand what will be done by the USRP and model it with block diagrams and a Matlab script. Report your findings. Use equations. For example, you can eventually model as follows:

(((x(t) + A) A1 cos(w1 t)) A2 cos(w2 t)

where x(t) represents your baseband signal (s1 or s2), and then you added a DC level A, multiplied by a cosine with amplitude A1 and the USRP multiplies by another cosine, with amplitude A2. Improve and expand this modeling, taking in account that the USRP receives a FDM signal. Describe the processing in block diagrams and include a companion Matlab scripts that emulates the USRP transmit processing. Explain how you control the modulation index in this scheme that includes the USRP.

The following tasks will be conducted together with the professor, during the class corresponding to the deadline, at LASSE.

Task 4: Use a signal generator to create an artificial AM signal and a spectrum analyzer to measure its modulation index. The manual of the (Agilent's) spectrum analyzer will inform how to deal with AM signals. Measure also the carrier power, noise floor, RBW and other values of interest. Include a screenshot of the analyzer in your report and a brief description of your setup. Make sure you can (at least roughly) understand all numerical values that are informed in each screenshot.

Task 5: Repeat Task 4 but now using a USRP to broadcast your binary file with the FDM signal. Try to observe the modulation index, bandwidth, etc. in the spectrum analyzer. Include screenshots in your report. Make sure you can (at least roughly) understand all numerical values that are informed in each screenshot.

Task 6: Use an off-the-shelf AM radio receiver to listen to your two stations. The RF frequency must coincide with the designed one. Do not blame the .grc script if that does not happen. Beforehand, work with the tutor to make sure things work at the demonstration (deadline) day.

Appendix – File read_complex_binary.m

% Copyright 2001 Free Software Foundation, Inc.

%

% This file is part of GNU Radio

%

% GNU Radio is free software; you can redistribute it and/or modify

% it under the terms of the GNU General Public License as published by

% the Free Software Foundation; either version 3, or (at your option)

% any later version.

%

% GNU Radio is distributed in the hope that it will be useful,

% but WITHOUT ANY WARRANTY; without even the implied warranty of

%...

function v = read_complex_binary (filename, count)

%% usage: read_complex_binary (filename, [count])

%%

%%  open filename and return the contents as a column vector,

%%  treating them as 32 bit complex numbers

%%

m = nargchk (1,2,nargin);

if (m)

usage (m);

end

if (nargin < 2)

count = Inf;

end

f = fopen (filename, 'rb');

if (f < 0)

v = 0;

else

t = fread (f, [2, count], 'float');

fclose (f);

v = t(1,:) + t(2,:)*i;

[r, c] = size (v);

v = reshape (v, c, r);

end

Appendix – File read_complex_binary.m

function ret = matlab_save_to_gnuradio(file_name, complex_signal)

% usage: matlab_save_to_gnuradio(file_name, complex_signal)

%

%   Save the vector of complex samples given as 'complex_signal' to a file

%   called 'file_name', use the specific format used by gnuradio.

% Written by Joary, LaPS, UFPA, 2014.

% Create vector to be written

vec_length = 2*length(complex_signal);

write_vec = zeros(1, vec_length);

% Organize data to be transmited

write_vec(1:2:vec_length) = real(complex_signal);

write_vec(2:2:vec_length) = imag(complex_signal);

% Write data to the file

f = fopen(file_name, 'w');

fwrite(f, write_vec, 'float32');

fclose(f);