|  | UNIVERSIDADE FEDERAL DO PARÁ INSTITUTO DE TECNOLOGIA PROGRAMA DE PÓS-GRADUAÇÃO EM ENGENHARIA ELÉTRICA DISCIPLINA: TRANSMISSÃO DIGITAL – PROF. ALDEBARO KLAUTAU |
| --- | --- |

## Laboratory:
AM modulation and demodulation on Matlab / Octave

Goal: Familiarize the reader with concepts of AM modulation and demodulation.

For all the simulations, assume the sampling frequency is Fs = 44100 Hz and the samples from the ADC were obtained with a quantizer with a step size corresponding to D=3 uV. In other words, if your ADC provides you the amplitudes as integer numbers x=[-2, -1, 3], they will correspond to [-6, -3, 9] microvolts, respectively. Use the correct units to specify waveforms, PSDs, etc. Take in account that Matlab's wavread function normalize the amplitudes and this is not indicated in our case because you will loose the information about the ADC levels that were used. In all simulations do not use IIR filters with order larger than 12 or FIR filters with order larger than 200.

Task 1: Collect three “original” signals of duration 12 seconds, each. Save them individually (3 files) as Microsoft RIFF (.WAV) files, using linear PCM, signed, 16 bits per sample. Two of them must be songs of your preference and the third one a recording with your own voice stating complete sentences, not unrelated words (avoid “testing, testing”).

a) Try to completely describe the contents of these original files using graphs with the axes properly labeled (clearly specify the units, such as seconds, Hz, etc.). Use the title to identify the signal. The following graphs should be obtained for each file, individually, and also for all of them concatenated as one signal (a total of 4 groups of graphs):

- time-domain waveforms, with axis in seconds and Volts (or milivolts).

- unilateral PSDs in dBm/Hz versus Hz assuming Fs/2 is the bandwidth.

- histograms of the signal amplitudes when represented by integers. Impose that the bins match the levels of a b-bits quantizer. For example, for b=8, force the 2b=256 bins to be centered at -128:127. The histograms will allow you to see how well you digitized your signal, whether or not you used almost all the bits of your ADC.

Task 2: Create channels to work with. Represent all channels by their impulse responses h[n] using at most M=500 samples for each h[n]. Normalize all filter responses such that their maximum amplitude value |H(f)|, considering all bandwidth [0, Fs/2] is approximately 10 log10(|H(f)|)=-6 dB.

a) Design a linear-phase passband FIR filter with the specifications in Fig. 1. Use any FIR design method you want. Call its impulse response h1.

Fig. 1 – Filter specifications (specs).

b) Design the second filter, h2, using the same specs of Fig. 1, but use an elliptic (Cauer) IIR filter. Simulations would be faster if you use its system function H(z), but to keep all the code based on impulse responses, store its impulse response h2.

c) Estimate the impulse response h3 of your sound system when connected in loopback using an audio cable. The textbook describes procedures for estimating h3.

d) Describe the three channels using detailed graphs (zoom as necessary):

- their magnitude frequency response in dB per Hz. All in one graph, with the proper legend.

- their group delay in seconds (or subunit) per Hz. All in one graph, with the proper legend. Discuss if the phase responses of each channel are linear or not, based on the group delay graphs.

Task 3: Design and simulate AM communication systems for all three channels. The transmit signals must be in Volts (or a subunit if you take proper care of this unit). Use envelope-based demodulation. Consider the need of resampling the signals to fit the channel bandwidth. Choose the carrier frequency(ies). Create a single Matlab script that processes all three files through all three channels, allowing the user to define the signal and the channel. This script must:

- use some sort of gain control to scale the receive signal and estimate the SNR between transmit and receive signals. Because we are not adding noise, this SNR is ideally infinite. Inform a table with the 9 resulting SNRs in dB. Try to maximize the SNRs by using improved filtering and synchronization in the receiver, and improved design (even limiting the transmit signal bandwidth has been tried). Never modify the transmit signals or channel, but for the gain control, you can assume that the receiver know the transmit signal power.

Task 4: Using a single signal composed of all 3 signals concatenated (you may have to process all the blocks chain), vary the modulation index h from 10% to 100% and observe the impact on SNR. Show a graph of SNR in dB versus h.

Task 5: Include an AWGN channel in your simulation just after the filter (try with h1, h2 and h3) with power corresponding to the one of thermal noise at room temperature (-174 dBm/Hz, as explained in http://en.wikipedia.org/wiki/Johnson%E2%80%93Nyquist_noise). Redo Task 4 considering AWGN. How much was the thermal noise power? What is the transmit signal power? What is the signal power after filtering (by h1, h2 or h3)? Did the thermal noise impacted the results? Why?

Task 6: (Bonus task, not mandatory) If time allows, investigate how “companding” can be used in AM to improve SNR. Redo the curve of SNR versus h when using companding. References for companding:

http://www.mathworks.com/help/comm/ref/compand.html

http://www.techopedia.com/definition/24121/companding

http://en.wikipedia.org/wiki/Companding

http://www.ti.com/lit/an/spra163a/spra163a.pdf

http://www.rle.mit.edu/isg/documents/ma_meng_thesis.pdf