|  | UNIVERSIDADE FEDERAL DO PARÁ INSTITUTO DE TECNOLOGIA PROGRAMA DE PÓS-GRADUAÇÃO EM ENGENHARIA ELÉTRICA DISCIPLINA: TRANSMISSÃO DIGITAL – PROF. ALDEBARO KLAUTAU |
| --- | --- |

## Laboratory:
FSK over the sound board

Goal: Familiarize the reader with the FSK modulation and signal processing using a computer sound board. More specifically, the user must implement a FSK modem and synchronization techniques to transmit in real-time a file between two computers. For starting, the V.21 modem can be used as a reference.

Introduction:

Using the textbook as a reference, the user is invited to get familiar with:

Application 6.1. A simplified version of an ITU-T V.21 FSK-based modem.

Application 6.2. QAM simulation.

Application 6.3. QAM over the sound board.

The script fsk_synchronous.m in the “Applications\FSKSynchronous” folder provides an example of synchronous FSK modulation and demodulation, without effectively transmitting sound.

The user must implement asynchronous demodulation. Do not assume that the carriers need to be regenerated at the receiver. Perform demodulation using filters.

The frequencies you choose do not need to be orthogonal, but if you want, you can read about orthogonality in FSK searching the textbook for the script:

MatlabOctaveBookExamples/ex_transforms_check_orthogonality.m

The user must transmit an image. For simplicity, it is assumed the image used in Application 6.1, which is binary. But if the user knows how to deal with images, it can be any image.

The user can adopt binary modulation or M-FSK. Always assume the transmitter is the caller modem and the received the called modem.

Both offline and online processing should used.

The transmission will be organized as follows:

- all images must have Pr x Pc pixels, where Pr and Pc are the number of rows and columns, respectively.

- Br is the number of bits representing a single row, and also denotes the size of a “packet”.

- When synchronized, the receiver will decode each packet and immediately (e.g., using Matlab's drawnow) draw the associated row.

- Np is the number of packets that compose a “frame”.

- A frame is composed of Np packets of Br bits, each, preceded by a synchronization sequence (or “sync”) with duration corresponding to No bits. In other words, the duration of a frame corresponds to Np x Br + No bits.

- The synchronization sequence, or preamble, is an M-sequence, as in Application 6.3.

- These parameters, the symbol rate, sampling frequency and all other parameters are defined by the user and must be reported.

Part I – FSK Modem using Offline Processing (only one computer is required)

In this part, the user will adopt a strategy similar to the one used in Application 6.3:

- Transmitter part:

a) Encode an image as an FSK signal, according to the described convention (define the overhead, etc).

* Include in the report the spectrogram for 4 seconds of signal

* Include in the report the transmit PSD

b) Save it as a WAV file.

- Transmission over the channel

c) Use a loopback cable and, with Audacity, transmit the previously recorded WAV and simultaneously record the received signal.

* Estimate the channel frequency response and show its graph. Describe the adopted estimation procedured.

* Estimate the channel group delay and show its graph.

* Estimate the channel intermodulation distortion (IMD) at least using as inputs the  frequencies adopted for representing the carriers.

- Receiver part:

d) Work with the WAV of the received signal and demodulate it using asynchronous FSK

* Show the transmit PSD

* Report the BER

* Show the reconstructed image and the error image

* Show a zoom of the time-domain received waveform with markers indicating the start of each packet and frame, after synchronization.

e) Put the Tx in loop, continuously repeating the transmit signal, and show on-the-fly the decoded figure at the receiver. Recall that each packet should be immediately displayed after decoding.

f) Not mandatory: use a video, as a sequence of images, and sequentially play the video.

Part II – FSK Modem using Online Processing (two computers are required)

a) With the Tx in loop on computer 1, use an audio cable to send the signal to computer 2. Develop code to have computer 2 working online with a circular buffer and not loosing samples.

b) Optimize the system to maximize the bit rate.

c) Study the clock offset between the two computers (between their DAC and ADC). Estimate this clock offset and explain your procedure.